Name: 	Joshua Philip E. Oropa						 Faculty: MB. Pacot

Section: JN1

Objective:

Instruction Set Format. Write the equivalent opcode for the following MIPS64 instructions. Assume all constants are in hexadecimal. For binary opcode, please place an appropriate space after each field in your binary opcode submission (10 PTS. each address).

| Address | Instruction | Opcode (Binary) | Opcode (Hex) |
| --- | --- | --- | --- |
| 00 | LW R1, 2000(R0) | 100011 00000 00001 0010000000000000 | 8C012000 |
| 04 | LWU R2, 2004(R0) | 100111 00000 00010 0010000000000100 | 9C022004 |
| 08 | SLT R3, R1, R2 | 000000 00001 00010 00011 00000 101010 | 0022182A |
| 0C | DADDIU R4, R1, R2 | 011001 00001 00100 0000000000000010 | 64240002 |
| 10 | DMUL R5, R1, R2 | 000000 00001 00010 00101 00010 011100 | 0022289C |
| 14 | DSLLV R6, R1, R2 | 000000 00010 00001 00110 00000 010100 | 00413014 |
| 18 | ANDI R5, R1, #FFFC | 001100 00001 00101 1111111111111100 | 3029FFFC |
| 1C | OR R6, R1, R2 | 000000 00001 00010 00110 00000 100101 | 00223025 |